# Risk Assessment: Worked Example

Fictional example: Oakfield Joinery Ltd, a 7-person joinery workshop. Use it as a model for tone and level of detail, then write your own. Copying someone else's assessment without thinking about your own workplace does not satisfy the law.

Company: Oakfield Joinery Ltd    Assessment date: 01/07/2026

Activity / area assessed: Workshop, machine shop and timber store

Assessor: Workshop manager, with input from all bench staff

Review date: 01/07/2027, or sooner if equipment or processes change

| Hazard | Who might be harmed and how | Existing controls | L | S | Risk | Further action, owner and due date |
| --- | --- | --- | --- | --- | --- | --- |
| Wood dust (inhalation) | All 6 workshop staff; hardwood dust can cause asthma and is a carcinogen | LEV extraction on fixed machines, FFP3 masks for sanding, weekly housekeeping | 3 | 4 | 12 | Service and test LEV (14-month cycle, due Aug 2026); add on-tool extraction for hand sanders. Owner: workshop manager. Due: 31/08/2026 |
| Circular saw and planer blades (contact) | Machine operators; severe cuts or amputation | Guards fitted and checked daily, push sticks in use, only trained staff on machines, braked motors | 2 | 5 | 10 | Refresher training for two newer staff; record checks in machine log. Owner: workshop manager. Due: 30/09/2026 |
| Manual handling of timber and finished units | All staff; back strain from boards up to 40kg and awkward finished joinery | Two-person lift rule over 25kg, trolleys and roller stands available | 3 | 3 | 9 | Add lifting section to induction; mark two-person items at goods-in. Owner: office manager. Due: 31/08/2026 |
| Slips and trips (offcuts, dust, trailing leads) | All staff and visitors | End-of-day sweep, cable drums mounted overhead, marked walkways | 3 | 2 | 6 | Move sweep to twice daily during busy periods. Owner: all staff, checked by workshop manager. Due: immediate |
| Fire (dust, finishes, offcut store) | All staff, visitors and neighbouring unit | Separate fire risk assessment in place; extinguishers serviced; flammables in COSHH cabinet | 2 | 5 | 10 | Review fire risk assessment alongside this one; keep offcut store below marked line. Owner: director. Due: 31/07/2026 |

Significant findings recorded because the company employs 5 or more people. Shared with all staff at the July toolbox talk. Signed: A. Oakfield (director), 01/07/2026.

Built by Digital Adaption. Free download from https://digitaladaption.co.uk/downloads/risk-assessment-template/